# Industrial Technology – Timber

## Tambour box – student resource

Student name:

Class:

Teacher name:

## Contents

Unit overview	3

Assignment overview	3

The design and production process	4

Unit requirements	5

Glossary	6

Research	8

Properties of timber	8

Cost and environmental impact	9

Timber types	10

Justification of timber	11

Existing designs	12

Existing design evaluation	13

Design ideas	15

Working drawings	17

Cutting list – tambour box	22

Costing exercise	23

Planning	24

School timetable	24

Time and action plan	25

Risk assessment	26

Job safety analysis	27

Construction process	29

Evaluation	33

## Unit overview

For this project, students make a small box with a tambour top that wraps around the back and connects underneath to the bottom of the drawer, so that when the drawer is opened the tambour section retracts at the same time. Students should have developed skills in using a variety of basic hand tools and machinery in the production of previous projects.

This project further develops the knowledge and skills of students in using the router with jigs to create complex items.

The dimensioned drawings for the tambour box and the jigs are provided and include suggestions for handle designs for the tambour section and the drawer front. Students will design and make their own handles and integrate them into their project.

## Assignment overview

Students work individually to produce their own tambour box from 15mm and 6mm thick timber. They will also need to work collaboratively with other students to negotiate the use of resources such as the router. Their box must operate smoothly with no obvious gaps or defects in the design.

Students are required to keep a portfolio of their research and construction as per the requirements outlined on the next page. They will use this with the experience of producing their box to evaluate its development and determine whether the design was successful.

## The design and production process

Throughout the study of Industrial technology, students use a design process and apply it to the development of their project.

The design and production process:

involves a sequence of organised steps which provide a solution to design needs and opportunities

may take a few seconds or minutes, such as when you select what clothes to wear, or may take years as in the case with the design of a motor vehicle

may involve one person or may involve many people

may be simple or complex, depending on the task

involves the designer questioning (or evaluating) throughout the process.

The sequence (or steps) in design processes may vary depending on design	Does your solution perform as it should?

Throughout the process, there should be ongoing evaluation and a review if required to improve the outcomes.

## Unit requirements

Students must submit the following items along with their finished project.

| Title | Description |
| --- | --- |
| Identifying and defining | - Completed glossary of terms. |
| Researching and planning | Properties of timber - Types of timber, including picture, characteristics, cost and availability - Contact details for a supplier for each type of timber - Justification of which timber would be best |
|  | Existing box designs - Research of at least 6 different box designs – including picture, description and evaluation of each |
|  | Generation of ideas for the handle - Sketch, dimension and label at least six design ideas for their handle. - Evaluate each idea to show the development of the chosen solution - Draw the selected design – include dimensions and details of construction. |
|  | Cutting list - Completed cutting list using the templates provided and costing for the timber selected |
|  | Time plan - A timeline for their project including consideration for, and evaluation of, their skill level, time available, selected material, tools and equipment required. Students should identify the skills they intend to develop through the construction of the project. |
|  | Construction flowchart - A flow chart of the construction process highlighting the key steps. - Sketches or pictures should be included with a description of what they did with ongoing evaluation identifying issues and their solutions. |
| Producing and implementing | - Students construct the tambour box following the flow chart and timelines. |
| Testing and evaluating | - A final evaluation using the scaffold that includes a justification for the response. Marks are awarded for justified responses. |

## Glossary

Complete the table below with definitions as you progress through the unit.

| Word | Definition |
| --- | --- |
| Tambour |  |
| Router |  |
| Jig |  |
| Guide bush |  |
| Down cut spiral bit |  |
| Routing template |  |
| Defects |  |
| Density |  |
| Strength |  |
| Workability |  |
| Grain |  |
| Figure |  |
| Texture |  |
| Cutting list |  |
| Risk assessment |  |
| JSA |  |
| Evaluation |  |

## Research

### Properties of timber

Choosing the correct timber for a job can be a difficult process due to the number of different species available. Considering the properties, application, environmental impact and cost of the different species available can guide your choice.

The properties and characteristics of timber are determined by the nature, type, size and arrangement of the cells which make up the wood. Properties of timber are also affected by moisture content and physical defects.

The main properties or working characteristics of timber that we need to consider are:

Colour

The colour of timber varies greatly. From pale grey to mid yellowy-brown, to deep red-brown. Your choice of colour will be guided by preference, availability of the specific timber species and the amount you are willing to spend. The aesthetic choice you make, based on the colour you want, needs to be balanced against these practicalities. Exotic timbers can be very expensive or may not be available in large sizes. If constructing a large project it could be worth investigating the use of inlays or veneers.

Grain

Grain is the alignment of the cells and fibres along the length of the wood. Grain can be described as straight, wavy, interlocking, cranky or sloping. Your choice of grain type depends on its availability and the desired appearance. Grain can also affect the strength and workability of the timber which we will look at later.

Texture

Texture describes how the sanded surface of the timber feels. Some timber feels soft and rough, which is known as coarse texture, while others feel very hard and glassy, which is known as a fine texture.

Figure

Figure refers to the appearance of the patterned surface. Figure is produced by variations in colour and the direction of the grain. Figure may also result from the presence of knots, growth rings and medullary rays. Figure can vary depending on the conversion technique used to harvest the timber from the tree. In some cases quarter sawing is said to produce the best figure.

Defects

Defects can be either natural or artificial and refer to a range of issues that occur with timber. Natural defects include knots, shakes, gum veins and burls and occur during the growth of the tree. Artificial defects such as spring, twist, bow, cup, collapse, felling shakes, and split occur during felling or due to defective seasoning if the timber dries out unevenly. Defects can affect the useability of the timber but can also be sought after to create particular grain effects such as using a burl to create veneers or bowls.

Density

Density is dependent on the size of the cells, the thickness of the cell walls and the number of substances such as gum or resin present. Density is usually quoted as light, medium or heavy for timber dried to 12% moisture content, however, the Equilibrium Moisture Content (EMC) of a piece of timber may vary above or below 12% depending on the local climatic conditions. A piece of timber may be considered more dense or less dense depending on its location. Light timbers such as Balsa have large cells with very thin cell walls, whereas dense timbers such as spotted gum have narrow cells with thick cell walls.

Strength

Strength refers to the ability of the timber to withstand forces or loads. Depending on the application, this may be important. For example, timber for a jewellery box may not need to be as strong as a timber beam supporting the roof of a house. Timber with non-straight grain can be more appealing aesthetically, but it can also significantly weaken the timber and this will need to be factored into the design. Construction timbers have ratings for their strength such as F5, F7 and MGP10.

Workability

Workability refers to how easy it is to shape the timber by cutting, machining, sanding, joining and gluing, and what it is generally like to work with. Timber with non-straight grain and higher density are generally less workable due to tool wear, lifting and handling.

### Cost and environmental impact

The cost of the timber must be weighed up against the other properties. Some timbers that have highly attractive features and colour can be very expensive compared to locally grown plantation timbers such as radiata pine.

Impact on the environment may also be considered. Some timbers originate in old growth rainforests that can take hundreds of years to regenerate. This can be weighed up against the use of plantation timbers that may have a lower or different impact on the environment.

### Timber types

Research two types of timbers and complete the table below.

Radiata pine has been included as an example.

| Timber Name and Supplier Details | Image and Cost | Characteristics and Working Properties |
| --- | --- | --- |
| Radiata Pine Local Hardware Store sales@termitetimber.com | Cost: $8.50/m for 140x19 | Radiata pine is a softwood whose availability and ease of use make it popular for all kinds of construction and decorative uses. The texture of radiata pine is fine, but uneven, and knots are common. The timber is fairly soft and has a low density, often with very wide annual growth rings. The sapwood is white to pale yellow, but often indistinguishable from the heartwood, which is light brown to yellow. Radiata pine is easy to work, apart from the knots, and it readily accepts finishes. |
|  | Cost: |  |
|  | Cost: |  |

### Justification of timber

Select the most suitable timber for your project and justify why it is the most suitable. You should consider the properties of the timber, such as colour, defects, density, strength and the cost.

Note: It may not be possible for you to use the timber you selected on this occasion as the school may already have purchased timber for this project.

### Existing designs

Research at least six design ideas of boxes with handles and outline whether the handle design would be suitable for your box. Place labelled and annotate pictures in the space below.

### Existing design evaluation

Evaluate each of the existing designs in the table below. In your evaluation, consider the following factors:

Appearance

Components

Processes

Technology

Function

| Design | Evaluation of design (describe positives and negatives) |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |
| 6 |  |

## Design ideas

In the space below and on the following page, sketch your own design ideas for your drawer handle based on the criteria discussed with your teacher. Consider some of the features from your research of existing designs.

### Working drawings

#### Router template

Note: the following is a full scale routing template for the internal groove that will house the tambour. It is designed to be used with a 3/8” router bush and a ¼” or 6.35mm down cut spiral router bit.

The outer dimensions should be 160mm long x 105mm wide, ensure that the dimensions are correct in case they have been scaled during printing.

## Cutting list – tambour box

Name:

Class:

Teacher:

| Part /component | Material | Length (mm) | Width (mm) | Thickness (mm) | Quantity |
| --- | --- | --- | --- | --- | --- |
| answer |  |  |  |  |  |
| answer |  |  |  |  |  |
| answer |  |  |  |  |  |
| answer |  |  |  |  |  |
| answer |  |  |  |  |  |
| answer |  |  |  |  |  |
| answer |  |  |  |  |  |
| answer |  |  |  |  |  |

## Costing exercise

Use the table below along with the per metre rate you researched, or as provided by your teacher, to calculate the cost of making your tambour box.

| Component | Cost ($/m) | Quantity | Total |
| --- | --- | --- | --- |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |

Total cost of components:

Labour cost $11.85/hour x _______________hours = ____________________

(This is approximately the wage earned by a first year Cabinetmaker apprentice)

| Items | Costs |
| --- | --- |
| Total cost for components + |  |
| Total cost of labour |  |
| Grand total to produce tambour box = |  |

## Planning

### School timetable

Colour in the appropriate squares in the table below to indicate your timber classes for the duration of this project. This will provide a visual representation of the amount of time you have to complete your tambour box.

You will also need to look at the school calendar to see how other activities such as sports days (S), public holidays (PH), excursions (E) or examination periods (EX) may affect the time you have available.

Term:

| Week | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Days | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Term:

| Week | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Days | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sports days (S)	 public holidays (PH) 	excursions (E) 	examination periods (EX)

### Time and action plan

On the following timeline, colour each square to indicate the steps you are going to take in constructing the tambour box. The coloured sections should show the amount of time allocated to each task and the order they should occur. Once you have completed your tambour box you will need to come back and evaluate whether this plan was successful and show where and why you needed to make changes.

| Actions | Wk1 | Wk2 | Wk3 | Wk4 | Wk5 | Wk6 | Wk7 | Wk8 | Wk9 | Wk10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dock material for sides to length |  |  |  |  |  |  |  |  |  |  |
| Setup jig and route groove in sides |  |  |  |  |  |  |  |  |  |  |
| Cut outside shape of sides |  |  |  |  |  |  |  |  |  |  |
| Cut shelf and base to size |  |  |  |  |  |  |  |  |  |  |
| Assemble carcase of sides, base and shelf |  |  |  |  |  |  |  |  |  |  |
| Cut drawer parts to size |  |  |  |  |  |  |  |  |  |  |
| Select and cut drawer joints |  |  |  |  |  |  |  |  |  |  |
| Assemble drawer and train fit in carcase |  |  |  |  |  |  |  |  |  |  |
| Cut and sand slats for tambour |  |  |  |  |  |  |  |  |  |  |
| Assemble tambour |  |  |  |  |  |  |  |  |  |  |
| Trial fit and then apply finish to all components |  |  |  |  |  |  |  |  |  |  |
| Final assembly of the finished box |  |  |  |  |  |  |  |  |  |  |

## Risk assessment

Risk assessment is a systematic examination of safety in the workplace. The aim is to:

Identify hazards

Assess injury severity and likelihood

Implement control measures to reduce or eliminate the risks

We do this to try and improve the overall safety for all people, whether that be in a school environment, another place of work or your home.

Risk assessments assess the safety hazards across the entire workplace and consider the controls necessary to minimise the risk for all the different activities taking place. In this unit you will be carrying out a Job Safety Analysis (JSA) which is very similar to a risk assessment, except you will be focusing on the construction of your box and the hazards associated with that.

| Empty cell | Very likely | Likely | Unlikely | Highly unlikely |
| --- | --- | --- | --- | --- |
| Fatality | High | High | High | Medium |
| Major injury | High | High | Medium | Medium |
| Minor injury | High | Medium | Medium | Low |
| Negligible injury | Medium | Medium | Low | Low |

The table above is a matrix used to assess the level of risk by considering the consequence against the likelihood of injury if exposed to the hazard. You will use this matrix to assess the level of risk for the hazards that you identify on the next page within the construction of your project. For each hazard, you will need to consider the potential control measures you can implement to try and eliminate or minimise each risk.

## Job safety analysis

| Title of operation: | Approved by: | Student name: |
| --- | --- | --- |
| Date: | Department: | Building/room: |

| Procedure (in steps) | Possible hazards | Risk level | Safety control |
| --- | --- | --- | --- |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |
| Empty cell |  |  |  |

## Construction process

Complete the following table to show the major steps in construction for your project. Use the same actions you listed in your time and action plan for each of the steps and elaborate on what you did, any problems encountered, and solutions you came up with.

| Step | Sketch/Image | Detailed description of process |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 |  |  |
| 11 |  |  |
| 12 |  |  |

## Evaluation

What were the main challenges/difficulties you faced when trying to make your project?

What other materials would be suitable for producing your project? What effect would they have?

Does your design function as intended? Is it suitable for its intended purpose and are there any modifications that would improve it?

Does your finished design match what you set out to build at the beginning of this unit? If it does, do you feel the design was successful? If it doesn’t, what happened along the way and how did that impact on your design?

What changes would you make to your design if you had to do it again?

Was your timeline accurate for what actually occurred? If there were changes, why did they occur and what could you have done differently to ensure that you stayed on track?